# Network Installation Practical Guide

## Project: ICT Laboratory Network Design (CDACC 2026)

## Unit: Practical Assessment 2 - Network Installation

Technician: Francis Mwangi Kienji
Date: March 10, 2026

### 1. PHYSICAL INSTALLATION & SAFETY (TASK 2.1)

Objective: Correctly terminate cables and ensure a safe working environment.

Cabling Instructions (T568B Standard):

- Tools Needed: Crimping Tool, Wire Stripper, RJ45 Connectors, Cat6 Cable.

- Color Code (Left to Right):

  - White-Orange

  - Orange

  - White-Green

  - Blue

  - White-Blue

  - Green

  - White-Brown

  - Brown

- Procedure: Strip the jacket, untwist and align the wires, trim them straight, insert into the RJ45 connector, and crimp firmly.

Safety Measures:

- Cable Management: Use cable ties or trunking to organize cables and prevent tripping hazards.

- PPE: Wear appropriate safety gear (e.g., antistatic wrist strap) when handling sensitive electronic components.

- Power Safety: Ensure all devices are powered off before connecting or disconnecting cables.

### 2. DEVICE CONFIGURATION (TASK 2.2)

Objective: Configure the network devices for connectivity and security.

2.1 Wi-Fi Router Configuration (WRT300N):

- Access GUI: Click the Router -> GUI tab.

- Basic Setup:

  - Internet Connection Type: Static IP (if provided) or DHCP.

  - Router IP: 192.168.1.1.

  - Subnet Mask: 255.255.255.0.

- DHCP Server Settings:

  - DHCP Server: Enabled.

  - Start IP Address: 192.168.1.100.

  - Maximum Number of Users: 50.

- Wireless Settings:

  - Network Name (SSID): ICT_LAB_WIFI.

  - Wireless Channel: 1, 6, or 11 (to avoid interference).

- Wireless Security:

  - Security Mode: WPA2 Personal.

  - Encryption: AES.

  - Passphrase: Kigumo@2026.

2.2 Network Printer Configuration:

- Access Config: Click the Printer -> Config tab -> FastEthernet0.

- IP Configuration: Select Static.

- IP Address: 192.168.1.10.

- Subnet Mask: 255.255.255.0.

- Default Gateway: 192.168.1.1 (Settings tab).

### 3. VERIFICATION (TASK 2.3)

Objective: Ensure all devices are correctly connected and communicating.

- Check Link Lights: Ensure all ports on the Switch and Router have green lights.

- Verify IP Lease: On a Student PC, go to Command Prompt and type ipconfig /renew to ensure it receives an IP from the 192.168.1.100+ range.

- Ping Test: Ping the Printer (192.168.1.10) from any PC to confirm connectivity.

Signature: __________________________
Technician: Francis Mwangi Kienji
Date: March 10, 2026