# Network Testing Practical Guide

## Project: ICT Laboratory Network Design (CDACC 2026)

## Unit: Practical Assessment 3 - Network Testing

Technician: Francis Mwangi Kienji
Date: March 10, 2026

### 1. CONNECTIVITY TESTING (TASK 3.1)

Objective: Verify that all devices can communicate across the network.

1.1 Ping Tests (Basic Connectivity):

- Test 1: Local Loopback

  - Open Command Prompt on Student PC 1.

  - Type ping 127.0.0.1.

  - Purpose: Verifies that the TCP/IP stack is correctly installed.

- Test 2: Default Gateway

  - Type ping 192.168.1.1.

  - Purpose: Verifies the connection to the Wi-Fi Router.

- Test 3: Network Printer

  - Type ping 192.168.1.10.

  - Purpose: Verifies access to shared resources.

- Test 4: Trainer PC

  - Type ping 192.168.1.2.

  - Purpose: Verifies inter-PC communication.

1.2 Traceroute (Path Analysis):

- Test 5: Internet Path

  - Type tracert 8.8.8.8.

  - Purpose: Shows the hop-by-hop path from the student PC to the internet (Google DNS).

### 2. NETWORK ANALYSIS & DIAGNOSTICS (TASK 3.2)

Objective: Use advanced tools to analyze network traffic and integrity.

2.1 Wireshark Packet Capture:

- Open Wireshark: Select the Ethernet interface.

- Filter: Type icmp in the filter bar and press Enter.

- Action: Perform a ping from Student PC 1 to the Printer.

- Analysis: Observe the Echo Request and Echo Reply packets. Note the source and destination IP addresses.

- Screenshot: Capture the Wireshark window showing the ICMP packets.

2.2 Cable Testing (Physical Layer):

- Tool: Network Cable Tester (Fluke or basic LED tester).

- Procedure: Connect both ends of the Cat6 cable to the tester.

- Result: Ensure all 8 LEDs light up in sequence (1-8). If any LED fails, the cable is faulty and must be re-terminated.

### 3. TEST RESULTS DOCUMENTATION (TASK 3.3)

Objective: Record findings for the examiner.

| Test Case | Destination | Result (Pass/Fail) | Observations |
| --- | --- | --- | --- |
| TC-01 | Loopback (127.0.0.1) | Pass | Stack is healthy. |
| TC-02 | Gateway (192.168.1.1) | Pass | Router is accessible. |
| TC-03 | Printer (192.168.1.10) | Pass | Shared resource found. |
| TC-04 | Trainer PC (192.168.1.2) | Pass | Communication successful. |
| TC-05 | Traceroute (8.8.8.8) | Pass | Path to internet is active. |
| TC-06 | Wireshark ICMP | Pass | Packets correctly routed. |
| TC-07 | Physical Cable | Pass | 8/8 pins terminated correctly. |

Signature: __________________________
Technician: Francis Mwangi Kienji
Date: March 10, 2026