# Packet Tracer Classroom Network Design Guide

## Project: ICT Laboratory Network Design (CDACC 2026)

## Unit: Practical Assessment 1 - Part 2

Technician: Francis Mwangi Kienji
Date: March 10, 2026

### 1. PHYSICAL NETWORK DIAGRAM (TASK 2.1)

Objective: Create a clear layout of the ICT Laboratory in the Physical Workspace.

Instructions:

- Open Physical View: Click the Physical tab in the top-left corner of Packet Tracer.

- Layout Placement:

  - Click on Home City -> Corporate Office -> Main Wiring Closet.

  - Place the 1 Switch and 1 Wi-Fi Router (WRT300N) in the rack.

- Device Placement:

  - Place 4 Student PCs at desks in a row.

  - Place 1 Trainer PC at the front desk.

  - Place 1 Network Printer near the Trainer PC.

- Cabling:

  - Use Copper Straight-Through cables to connect all 5 PCs and the Printer to the Switch ports (FastEthernet0/1 to 0/6).

  - Connect the Switch to one of the Ethernet LAN ports on the Wi-Fi Router.

- Labeling: Use the Note Tool (N) to label each PC (e.g., "Student PC 1", "Trainer PC") and the Router ("ICT_LAB_WIFI").

- Screenshot: Capture the layout and save it as “physical network” in your project folder.

### 2. LOGICAL NETWORK DIAGRAM (TASK 2.2)

Objective: Configure the network logic, IP addressing, and connectivity.

Instructions:

- Topology Identification: This is a Star Topology (all devices connect to a central switch).

- Addressing Plan:

| Device | Configuration Type | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Wi-Fi Router | Static | 192.168.1.1 | 255.255.255.0 | N/A |
| Trainer PC | Static | 192.168.1.2 | 255.255.255.0 | 192.168.1.1 |
| Network Printer | Static | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| Student PCs | DHCP | Automatic | 255.255.255.0 | 192.168.1.1 |

- Device Configuration:

  - Wi-Fi Router: Click the Router -> GUI tab. Set the LAN IP to 192.168.1.1. Enable the DHCP Server (Start IP: 192.168.1.100).

  - Network Printer: Click the Printer -> Config tab -> FastEthernet0. Set IP to 192.168.1.10 and Subnet Mask to 255.255.255.0.

  - Student PCs: Click each PC -> Desktop -> IP Configuration -> Select DHCP.

- Testing Connectivity (Ping):

  - Open the Command Prompt on any Student PC.

  - Ping Gateway: Type ping 192.168.1.1.

  - Ping Printer: Type ping 192.168.1.10.

  - Ping Internet Cloud: Connect a Cloud-PT to the Router's Internet port and ping its IP (e.g., 8.8.8.8 if configured).

- Screenshots:

  - Capture the Ping results (Command Prompt window).

  - Capture the Logical Diagram (Workspace view).

  - Save these in your folder as “logical diagram”.

Signature: __________________________
Technician: Francis Mwangi Kienji
Date: March 10, 2026